Vision of the institute

Development of academically excellent, culturally vibrant, socially responsible and globally competent human resources.

Mission of the institute

To keep pace with advancements in knowledge and make the students competitive and capable at the global level.

To create an environment for the students to acquire the right physical, intellectual, emotional and moral foundations to shine as torch bearer of tomorrow’s society.

To strive to attain ever-higher benchmarks of educational excellence.

Vision of the Department

The department will be recognised for its value based teaching, associated activities pertaining to research and entrepreneurship.

Mission of the Department

- To provide quality education through faculty and state of art infrastructure

- To identify the current problems in society pertaining to Civil Engineering disciplines and to address them effectively and efficiently

- To inculcate the habit of research and entrepreneurship in our graduates to address current infrastructure needs of society

PEO’s

Graduates who complete their UG through our institute will be,

PEO 1- Engaged in professional practices, such as construction, environmental, geotechnical, structural, transportation, water resource engineering by using technical, communication and management skills.

PEO 2- Engaged in higher studies and research activities in various civil engineering fields and life time commitment to learn ever changing technologies to satisfy increasing demand of sustainable infrastructural facilities.

PEO 3- Serve in a leadership position in any professional or community organization or local or state engineering board

PEO 4- Registered as professional engineer or developed a strong ability leading to professional licensure being an entrepreneur.

PSO’s

PSO 1 – To apply science, mathematics and mechanics to solve problems in engineering realm

PSO 2 – To analyse the techniques, skills and modern engineering tools necessary for engineering practices

PSO 3 – To develop ability to function as a leader and a team player in multidisciplinary teams

PSO 4 – To recognize of the need for and an ability to engage in research and life-long learning for developing sustainable construction practices

PSO 5 – To design and conduct experiments as well as to analyse and interpret data

Unit – 3

Treatment / Processing Techniques

Structure

3.0 Introduction

3.1 Objectives

3.2 Component separation

3.3 Mechanical volume and size reduction

3.4 Chemical volume reduction

3.5 Biological processing

3.6 Assignment questions

3.7 Outcomes

3.8 Further Reading

3.0 Introduction

The processing of wastes helps in achieving the best possible benefit from every functional element of the solid waste management (SWM) system and, therefore, requires proper selection of techniques and equipment for every element. Accordingly, the wastes that are considered suitable for further use need to be paid special attention in terms of processing, in order that we could derive maximum economical value from them. The purposes of processing, essentially, are

- Improving efficiency of SWM system: Various processing techniques are available to improve the efficiency of SWM system. For example, before waste papers are reused, they are usually baled to reduce transporting and storage volume requirements. In some cases, wastes are baled to reduce the haul costs at disposal site, where solid wastes are compacted to use the available land effectively.

- Recovering material for reuse: Usually, materials having a market, when present in wastes in sufficient quantity to justify their separation are most amenable to recovery and recycling. Materials that can be recovered from solid wastes include paper, cardboard, plastic, glass, ferrous metal, aluminium and other residual metals.

- Recovering conversion products and energy: Combustible organic materials can be converted to intermediate products and ultimately to usable energy. This can be done either through incineration, pyrolysis, composting or bio-digestion.

3.1 Objectives

- Identify the purpose of waste processing;

- Explain the processing techniques for reducing the volume and size of wastes;

- Carry out separation of various components;

- Discuss the need for dewatering and drying of wastes;

- Assess technical viability of various processing techniques.

3.2 Component separation:

Component separation is a necessary operation in which the waste components are identified and sorted either manually or mechanically to aid further processing. This is required for the:

- Recovery of valuable materials for recycling;

- Preparation of solid wastes by removing certain components prior to incineration, energy recovery, composting and biogas production.

The most effective way of separation is manual sorting in households prior to collection. In many cities (e.g., Bangalore, Chennai, etc., in India), such systems are now routinely used. The municipality generally provides separate, easily identifiable containers into which the householder deposits segregated recyclable materials such as paper, glass, metals, etc. Usually, separate collections are carried out for the recyclable material. At curb side, separate areas are set aside for each of the recyclable materials for householders to deliver material – when there is no municipal collection system. In case the separation is not done prior to collection, it could be sorted out through mechanical techniques such as air separation, magnetic separation, etc., to recover the wastes.

Air separation:

Air separation is primarily used to separate lighter materials from heavier ones. The lighter material may include plastics, paper and paper products and other organic materials. In this technique, the heavy fraction is removed from the air classifier to the recycling stage or to land disposal, as appropriate. The light fraction may be used, with or without further size reduction, as fuel for incinerators or as compost material. There are various types of air classifiers commonly used, some of which are listed below:

Conventional chute type:

In this type, when the processed solid wastes are dropped into the vertical chute, the lighter material is carried by the airflow to the top while the heavier materials fall to the bottom of the chute. The control of the percentage split between the light and heavy fraction is accomplished by varying the waste loading rate, airflow rate and the cross section of chute. A rotary air lock feed mechanism is required to introduce the shredded wastes into the classifier.

Zigzag air classifier:

Shredded wastes are introduced at the top of the column at a controlled rate, and air is introduced at the bottom of the column. As the wastes drop into the air stream, the lighter fraction is fluidised and moves upward and out of column, while the heavy fraction falls to the bottom. Best separation can be achieved through proper design of the separation chamber, airflow rate and influent feed rate.

Open inlet vibrator type:

In this type of air classifier, the separation is accomplished by a combination of the following actions:

Vibration: This helps to stratify the material fed to the separator into heavy and light components. Due to this agitation, the heavier particles tend to settle at the bottom as the shredded waste is conveyed down the length of the separator.

Inertial force: In this action, the air pulled in through the feed inlet imparts an initial acceleration to the lighter particle, while the wastes travel down the separator as they are being agitated.

Air pressure: This action refers to the injection of fluidising air in two or more high velocity and low mass flow curtains across the bed. A final stripping of light particles is accomplished at the point where the heavy fraction discharges from the elutriators. It has been reported that the resulting separation is less sensitive to particle size than a conventional vertical air classifier, be it of straight or zigzag design. An advantage of this classifier is that an air lock feed mechanism is not required and wastes are fed by gravity directly into the separator inlet.

Selection of air separation equipment:

The factors that are to be considered for selecting air separation equipment include the following:

- Characteristics of the material produced by shredding equipment including particle size, shape, moisture content and fibre content.

- Material specification for light fraction.

- Methods of transferring wastes from the shredders to the air separation units and feeding wastes into the air separator.

- Characteristics of separator design including solids-to-air ratio, fluidising velocities, unit capacity, total airflow and pressure drop.

- Operational characteristics including energy requirement, maintenance requirement, simplicity of operation, proved performance and reliability, noise output, and air and water pollution control requirements.

- Site considerations including space and height access, noise and environmental limitations.

Magnetic separation

The most common method of recovering ferrous scrap from shredded solid wastes involves the use of magnetic recovery systems. Ferrous materials are usually recovered either after shredding or before air classification. When wastes are mass-fired in incinerators, the magnetic separator is used to remove the ferrous material from the incinerator residue. Magnetic recovery systems have also been used at landfill disposal sites.

Equipment used for magnetic separation Various types of equipment are in use for the magnetic separation of ferrous materials. The most common types are the following:

Suspended magnet:

In this type of separator, a permanent magnet is used to attract the ferrous metal from the waste stream. When the attracted metal reaches the area, where there is no magnetism, it falls away freely. This ferrous metal is then collected in a container.

This type of separation device is suitable for processing raw refuse, where separators can remove large pieces of ferrous metal easily from the waste stream.

Magnetic pulley:

This consists of a drum type device containing permanent magnets or electromagnets over which a conveyor or a similar transfer mechanism carries the waste stream. The conveyor belt conforms to the rounded shape of the magnetic drum and the magnetic force pulls the ferrous material away from the falling stream of solid waste.

Selection of magnetic separation equipment:

We must consider the following factors in the selection of magnetic separation equipment:

- Characteristics of waste from which ferrous materials are to be separated (i.e., the amount of ferrous material, the tendency of the wastes to stick to each other, size, moisture content, etc.)

- Equipment used for feeding wastes to separator and removing the separated waste streams.

- Characteristics of the separator system engineering design, including loading rate, magnet strength, and conveyor speed, material of construction, etc.

- Operational characteristics, including energy requirements, routine and specialised maintenance requirements, and simplicity of operation, reliability, noise output, and air and water pollution control requirements.

- Locations where ferrous materials are to be recovered from solid wastes.

- Site consideration, including space and height, access, noise and environmental limitations.

Screening:

Screening is the most common form of separating solid wastes, depending on their size by the use of one or more screening surfaces. Screening has a number of applications in solid waste resource and energy recovery systems. Screens can be used before or after shredding and after air separation of wastes in various applications dealing with both light and heavy fraction materials. The most commonly used screens are rotary drum screens and various forms of vibrating screens.

Note that rotating wire screens with relatively large openings are used for separation of cardboard and paper products, while vibrating screens and rotating drum screens are typically used for the removal of glass and related materials from the shredded solid wastes.

Selection of screening equipment:

The various factors that affect the selection of screens include the following:

- Material specification for screened component.

- Location where screening is to be applied and characteristics of waste material to be screened, including particle size, shape, bulk, density and moisture content.

- Separation and overall efficiency.

- Characteristics screen design, including materials of construction, size of screen openings, total surface screening area, oscillating rate for vibrating screens, speed for rotary drum screens, loading rates and length.

- Operational characteristics, including energy requirements, maintenance requirements, simplicity of operation, reliability, noise output and air and water pollution control requirements.

- Site considerations such as space and height access, noise and related environmental limitations.

Other separation techniques:

Besides the mechanical techniques we studied earlier for segregating wastes, there are others. A description of some of these other separation techniques is given below:

Hand-sorting or previewing:

Previewing of the waste stream and manual removal of large sized materials is necessary, prior to most types of separation or size reduction techniques. This is done to prevent damage or stoppage of equipment such as shredders or screens, due to items such as rugs, pillows, mattresses, large metallic or plastic objects, wood or other construction materials, paint cans, etc.

Inertial separation:

Inertial methods rely on ballistic or gravity separation principles to separate shredded solid wastes into light (i.e., organic) and heavy (i.e., inorganic) particles.

Ballistic Inertial Separator:

Inclined Conveyor Separator:

Flotation:

In the flotation process, glass-rich feedstock, which is produced by screening the heavy fraction of the air-classified wastes after ferrous metal separation, is immersed in water in a soluble tank. Glass chips, rocks, bricks, bones and dense plastic materials that sink to the bottom are removed with belt scrappers for further processing. Light organic and other materials that float are skimmed from the surface. These materials are taken to landfill sites or to incinerators for energy recovery. Chemical adhesives (flocculants) are also used to improve the capture of light organic and fine inorganic materials.

Optical sorting:

Optical sorting is used mostly to separate glass from the waste stream, and this can be accomplished by identification of the transparent properties of glass to sort it from opaque materials (e.g., stones, ceramics, bottle caps, corks, etc.) in the waste stream. Optical sorting involves a compressed air blast that removes or separates the glasses – plain or coloured. Optical sorting machinery is, however, complex and expensive

3.3 Mechanical volume and size reduction:

Mechanical volume and size reduction is an important factor in the development and operation of any SWM system. The main purpose is to reduce the volume (amount) and size of waste, as compared to its original form, and produce waste of uniform size.

Volume reduction or compaction: (Jan 10/Dec11/Jan 13)

Volume reduction or compaction refers to densifying wastes in order to reduce their volume. Some of the benefits of compaction include:

- Reduction in the quantity of materials to be handled at the disposal site;

- Improved efficiency of collection and disposal of wastes;

- Increased life of landfills;

- Economically viable waste management system.

However, note the following disadvantages associated with compaction:

- Poor quality of recyclable materials sorted out of compaction vehicle;

- Difficulty in segregation or sorting (since the various recyclable materials are mixed and compressed in lumps);

- Bio-degradable materials (e.g., leftover food, fruits and vegetables) destroy the value of paper and plastic material.

Equipment used for compaction:

Based on their mobility, we can categorise the compaction equipment used in volume reduction under either of the following:

Stationary equipment:

This represents the equipment in which wastes are brought to, and loaded into, either manually or mechanically. In fact, the compaction mechanism used to compress waste in a collection vehicle, is a stationary compactor. According to their application, stationary compactors can be described as light duty, commercial or light industrial, heavy industrial and transfer station compactors. Usually, large stationary compactors are necessary, when wastes are to be compressed into:

- Steel containers that can be subsequently moved manually or mechanically;

- Chambers where the compressed blocks are banded or tied by some means before being removed;

- Chambers where they are compressed into a block and then released and hauled away untied;

- Transport vehicles directly.

Movable equipment: This represents the wheeled and tracked equipment used to place and compact solid wastes, as in a sanitary landfill.

Types of Compaction Equipment

Solid waste generation points

- Vertical- Vertical compaction ram may be used; may be mechanically or hydraulically operated, usually hand-fed; wastes compacted into corrugated box containers, or paper or plastic bags; used in medium and high-rise apartments.

- Rotary- Ram mechanism used to compact waste into paper or plastic bags on rotating platform, platform rotates as containers are filled; used in medium and high-rise apartments.

- Bag or extruder- Compactor can be chute fed; either vertical or horizontal rams; single or continuous multi-bags; single bag must be replaced and continuous bags must be tied off and replaced; used in medium and high-rise apartments.

- Under counter- Small compactors used in individual residences and apartment units; wastes compacted into special paper bags; after wastes are dropped through a panel door into a bag and door is closed, they are sprayed for odour control; button is pushed to activate compaction mechanism.

Collection

- Stationary/packers- Collection vehicles equipped with compaction mechanism.

Transfer and/or processing station

- Stationary/transfer trailer- Transfer trailer, usually enclosed, equipped with self-contained compaction mechanism.

- Stationary low pressure- Wastes are compacted into large containers.

- Stationary high pressure- Wastes are compacted into dense bales or other forms.

Disposal site

- Movable wheeled or tracked equipment- Specially designed equipment to achieve maximum compaction of wastes.

- Stationary/track mounted- High-pressure movable stationary compactors used for volume reduction at a disposal site.

Compactors:

According to their compaction pressure, we can divide the compactors used at transfer stations as follows:

Low-pressure (less than 7kg/cm2) compaction:

This includes those used at apartments and commercial establishments, bailing equipment used for waste papers and cardboards and stationary compactors used at transfer stations. In low-pressure compaction, wastes are compacted in large containers. Note that portable stationary compactors are being used increasingly by a number of industries in conjunction with material recovery options, especially for waste paper and cardboard.

High-pressure (more than 7kg/cm2) compaction:

Compact systems with a capacity up to 351.5 kg/cm2 or 5000 lb/in2 come under this category. In such systems, specialised compaction equipment is used to compress solid wastes into blocks or bales of various sizes. In some cases, pulverised wastes are extruded after compaction in the form of logs. The volume reduction achieved with these high-pressure compaction systems varies with the characteristics of the waste. Typically, the reduction ranges from about 3 to 1 through 8 to 1.

Selection of compaction equipment:

To ensure effective processing, we need to consider the following factors, while selecting compaction equipment:

- Characteristics such as size, composition, moisture content, and bulk density of the waste to be compacted.

- Method of transferring and feeding wastes to the compactor, and handling.

- Potential uses of compacted waste materials.

- Design characteristics such as the size of loading chamber, compaction pressure, compaction ratio, etc.

- Operational characteristics such as energy requirements, routine and specialised maintenance requirement, and simplicity of operation, reliability, noise output, and air and water pollution control requirement.

- Site consideration, including space and height, access, noise and related environmental limitations.

Size reduction or shredding (Jan 10/Dec11/Jan 13)

This is required to convert large sized wastes (as they are collected) into smaller pieces. Size reduction helps in obtaining the final product in a reasonably uniform and considerably reduced size in comparison to the original form. But note that size reduction does not necessarily imply volume reduction, and this must be factored into the design and operation of SWM systems as well as in the recovery of materials for reuse and conversion to energy. In the overall process of waste treatment and disposal, size reduction is implemented ahead of:

- Land filling to provide a more homogeneous product. This may require less cover material and less frequent covering than that without shredding.

- Recovering materials from the waste stream for recycling.

- Baling the wastes – a process sometimes used ahead of long distance transport of solid wastes – to achieve a greater density.

- Making the waste a better fuel for incineration waste energy recovery facilities.

- Reducing moisture, i.e., drying and dewatering of wastes.

Size reduction equipment:

| Type | Mode of action | Application |
| --- | --- | --- |
| Small grinders | Grinding, mashing | Organic residential solid wastes |
| Chippers | Cutting, slicing | Paper, cardboard, tree trimmings, yard waste, wood, plastics |
| Large grinders | Grinding, mashing | Brittle and friable materials, used mostly in industrial operation |
| Jaw crushers | Crushing, breaking | Large solids |
| Rasp mills | Shredding, tearing | Moistened solid wastes |
| Shredders | Shearing, tearing | All types of municipal wastes |
| Cutters, Clippers | Shearing, tearing | All types of municipal wastes |
| Hammer mills | Breaking, tearing, cutting, crushing | All types of municipal wastes, most commonly used equipment for reducing size and homogenizing composition of wastes |
| Hydropulper | Shearing, tearing | Ideally suited for use with pulpable wastes, including paper, wood chips. Used primarily in the papermaking industry. Also used to destroy paper records |

The most frequently used shredding equipments are the following:

Hammer mill:

These are used most often in large commercial operations for reducing the size of wastes. Hammer mill is an impact device consisting of a number of hammers, fastened flexibly to an inner disk, which rotates at a very high speed:

Solid wastes, as they enter the mill, are hit by sufficient force, which crush or tear them with a velocity so that they do not adhere to the hammers. Wastes are further reduced in size by being struck between breaker plates and/or cutting bars fixed around the periphery of the inner chamber. This process of cutting and striking action continues, until the required size of material is achieved and after that it falls out of the bottom of the mill.

Hydropulper:

Solid wastes and recycled water are added to the hydropulper. The high-speed cutting blades, mounted on a rotor in the bottom of the unit, convert pulpable and friable materials into slurry with a solid content varying from 2.5 to 3.5%. Metal, tins, cans and other non-pulpable or non-friable materials are rejected from the side of the hydropulper tank. The rejected material passes down a chute that is connected to a bucket elevator, while the solid slurry passes out through the bottom of the pulper tank and is pumped to the next processing operation.

The factors that decide the selection of size reduction equipment include the following:

- The properties of materials before and after shredding.

- Size requirements for shredded material by component.

- Method of feeding shredders, provision of adequate shredder hood capacity (to avoid bridging) and clearance requirement between feed and transfer conveyors and shredders.

- Types of operation (continuous or intermittent).

- Operational characteristics including energy requirements, routine and specialised maintenance requirement, simplicity of operation, reliability, noise output, and air and water pollution control requirements.

- Site considerations, including space and height, access, noise and environmental limitations.

- Metal storage after size reduction for the next operation.

3.4 Chemical volume reduction:

Chemical volume reduction is a method, wherein volume reduction occurs through chemical changes brought within the waste either through an addition of chemicals or changes in temperature. Incineration is the most common method used to reduce the volume of waste chemically, and is used both for volume reduction and power production. These other chemical methods used to reduce volume of waste chemically include pyrolysis, hydrolysis and chemical conversions.

3.5 Biological processing:

Composting:

Composting involves the biological decomposition of organic materials under controlled conditions that allow for the development of an end product that is biologically stable and free of viable pathogens and can be applied to land beneficially.

Organic matter + O2 + aerobic bacteria ---->CO2 + NH3 + H2O + other end products + energy

Bio-gasification:

Biogas is a mixture of gases composed of methane (CH4) 40 – 70 vol.%, carbon dioxide (CO2) 30 – 60 vol.%, other gases 1 – 5 vol.% including, hydrogen (H2) 0 – 1 vol.% and hydrogen sulphide (H2S) 0 – 3 vol.%. It originates from bacteria in the process of bio-degradation of organic material under anaerobic (without air) conditions. The natural generation of biogas is an important part of the biogeochemical carbon cycle.

3.6 Recommended Questions

- Explain the factors to be considered in evaluating onsite process techniques

- Explain the following

  - Mechanical volume reduction 2. Mechanical size reduction

- Explain briefly, the processing techniques involved in the treatment of MSW as well as material recovery

3.7 Outcomes

- Apply steps in solid waste management-waste reduction at source, collection techniques, materials and resource recovery/recycling

  - Further Reading

1	http://nptel.ac.in/courses/120108005/module5/lecture5.pdf

- http://www.seas.columbia.edu/earth/wtert/sofos/nawtec/1990-National-Waste-Processing-Conference/1990-National-Waste-Processing-Conference-42.pdf